Name: ___________________________ Class: ___________________________

# Lesson 2: Worksheet 1

Answer the following questions as you view the accompanying slide presentation.

- If a load has a resistance of 5 ohms, how much current will it draw from a 120-volt power Supply?

- If a load is drawing 1.5 amps from a 120-volt supply, what is the load’s resistance?

- A 23-ohm load is drawing 5.0 amps. What is the supply voltage?

- If an appliance has a cord that can only handle 10 amps, how much voltage needs to be applied to it to deliver 2400 watts?

- A 120-volt toaster draws 1200 watts. How many amps is this?

- How many watts can a 15-amp circuit deliver before tripping the 120V circuit breaker?

- How many amps does a 60-watt light bulb draw from a 120-volt supply?

- If a load draws 42 amps from a 120-volt supply, how many amps will it draw from a 240-volt supply to receive the same power?

- A cell phone charger claims to draw 0.35 amps from a 120-volt outlet. How many watts is this?

- A 50-foot length of wire has a resistance of 0.08 ohms. If 10 amps are flowing through the wire, what is the voltage loss across this length of wire?