Name: ___________________________ Class: ___________________________

# Unit Three Exam

- Name 2 organizations that create welding codes for WPS’s?

1. ____________________________________________________________

2. ____________________________________________________________

- Fill in the blank of the missing steps to qualify a Welding Procedure and Certifying a Welder.

1. ____________________________________

2. Write a Welding Procedure Specification

3. Qualify welding procedure

4. Qualify welder

5. Certify welder

- Analyzing Design includes all the following except which one?

a) Joint design

b) Weld type

c) Filler wire size being used

d) Location of weld

- A Certified Welding Inspector or Welding Engineer has to create a certified WPS.

a) True

b) False

- A welding procedure can pass as long as only 1 test fails and no more than 1.

a) True

b) False

- A Certified Welding Inspector does not need to be present while a welder is testing to ensure that the test was completed in accordance to the WPS.

a) True

b) False

READING A WPS

Name the following positions by their alternative number:

- Flat Position = ________________________

- Horizontal = __________________________

Name the following positions with their alternative name:

- 4 = _____________________________

- 3 = _____________________________

- The “G” in a “1G” position would be talking about what type of weld?

a) Overhead

b) Groove

c) V-groove

d) Vertical

- The “F” in a “4F” position would be talking about what type of weld?

a) First

b) Four

c) Flat

d) Fillet

- What is not a type of transfer mode for GMAW?

a) Short circuit

b) Pulse

c) Spray

d) Groove

e) Globular

- The number of welding passes would be found in this area on the WPS.

a) Base Metals & Filler Metals

b) Shielding Gas

c) Technique

d) Preheat

- As long as you follow at least half of the WPS, you will be able to create a quality, certified weld.

a) True

b) False

NON-DESTRUCTIVE TESTING

- What type is not a Non-Destructive Examination?

a) Visual

b) Liquid Penetrant

c) Magnetic Particle

d) Hypersonic

e) Radiographic

- When is a Visual Examination done?

a) After by an instructor

b) After, by both an instructor and the welder themselves

c) Before, during, after the weld is made

d) Never, a visual exam doesn’t do any good

- Name 4 of the 8 Visual Examination Tools

1. ___________________________________

2. ___________________________________

3. ___________________________________

4. ___________________________________

- Magnetic Particle Examination can be used with Aluminum.

a) True

b) False

- Ultrasonic Examination uses all the following except what?

a) Uses sound waves to detect surface and subsurface flaws

b) A transducer

c) Little to no skill level needed to examine because the ultrasound does it for you

- What type of rays penetrates metal to show Radiographic Examination results?

a) Gamma rays

b) Subsurface rays

c) Tungsten rays

d) Radiographic rays

DESTRUCTIVE TESTING

- Destructive Testing is used to test which of the following?

a) Welded products

b) Welds

c) Filler metals

d) Base metals

e) Welding Procedure Specifications (WPS)

f) All of the above

- Name 3 of the 6 types of Destructive Testing

1. ________________________________

2. ________________________________

3. ________________________________

- Tension Testing measures what?

a) How good root penetration is

b) Heat Input to a weld

c) How much a material can withstand being pulled apart

d) Toughness of base metals

e) Toughness of filler metals

- Impact Testing is typically performed in a lab environment.

a) True

b) False

- Bend Testing is commonly used to qualify welders and welding procedures (WPS).

a) True

b) False

- Explain what usually happens in a Break Test

________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

- The Nick Break Test usually tests specimens from?

a) A fillet weld

b) A groove weld

c) A lap joint

d) A t-joint

- Etch Testing is used to?

a) Inspect the work angle a welder used during a weld

b) Inspect the travel angle a welder used during a weld

c) Inspect for soundness of welds and metals

- Etch Testing has two types of tests, what are they?

a) Macro & Micro

b) Macro & Minnie

c) Micro & Major

d) Minor & Major