# Student Quiz - Welding Equipment

Name:

- What polarity is GMAW typically performed for the best results?

  - Direct current electrode positive

  - Direct current electrode negative

  - Alternating current

  - Polarity makes no difference in the quality of a weld

- What terminal does the wire feeder or gun plug into the power source to obtain direct current electrode positive (DCEP) polarity?

  - Wherever the connector fits

  - Negative terminal

  - Positive terminal

  - Alternating terminal

- Why is the work cable and clamp NOT considered a ground?

  - It connects to the work piece, not the ground

  - It completes a circuit

  - It interrupts a circuit

  - It makes the weld flow

- What are the parts of a wire feeder?

  - Drive rolls, tension adjustment, wire spool

  - Drive rolls, guides, wire spool

  - Drive rolls, tension adjustment, liner

  - Drive rolls, guides, tension adjustment

- What are the parts of a GMAW gun?

  - Liner, diffuser, contact tip, nozzle, trigger

  - Liner, diffuser, guides, nozzle, trigger

  - Liner, guides, drive rolls, contact tip

  - Liner, contact tip, tension adjustment, nozzle, trigger

- When the trigger is depressed, what comes out of the gun?

  - Welding wire

  - Shielding gas

  - The arc

  - Both a and b

- In what type of system will shielding gas be delivered to the welding station through a piping system?

  - Cylinder system

  - Manifold system

  - Shielding gas system

  - Uniform system

- What does a pressure gauge do?

  - Measure the amount of shielding gas pressure going to the weld

  - Measure the amount of shielding gas pressure in the cylinder

  - Regulate the amount of shielding gas pressure going to the weld

  - Regulate the amount of shielding gas pressure in the cylinder

- Which of the following statements is FALSE about inverter technology?

  - Welders with inverter technology can only weld on thin material

  - Welders with inverter technology use less power

  - Welders with inverter technology are smaller

  - Welders with inverter technology have more capabilities

- If you change the diameter of the wire, what else needs to be changed?

  - Drive rolls, guides, nozzle, and liner

  - Drive rolls, diffuser, contact tip, and nozzle

  - Drive rolls, contact tip, guides, and nozzle

  - Drive rolls, guides, contact tip, and liner

- The lowest the average tensile strength can go from what amount to 120?

  - 40

  - 50

  - 30

  - 60

- After you have done a Stick weld, you should use what to clean off the slag?

  - Screwdriver

  - Chipping hammer

  - Water

  - Rub it off with a cloth

- True	False	When your SMAW/Stick machine is on and your ground clamp is in place the electrode (or electrode holder if no electrode is in place) is considered HOT!

- True	False	Slag is something that you must remove in SMAW/Stick welding before you can weld on top of the previous weld?

- True	False	You should always turn off your SMAW/Stick welding machine before plugging it into the wall?