# Student Quiz - Weld Defects

Name:

- What is the term for gas pockets formed in a weld bead?

  - Spatter

  - Porosity

  - Crater crack

  - Undercut

- What direction does a transverse crack run?

  - Parallel to the axis of the weld bead

  - At a 450 angle to the axis of the weld bead

  - Perpendicular to the axis of the weld bead

  - In the root of the bead only

- What direction does a longitudinal crack run?

  - Parallel to the axis of the weld bead

  - At a 450 angle to the axis of the weld bead

  - Perpendicular to the axis of the weld bead

  - In the root of the bead only

- How are cracks formed?

  - Hitting the weld bead with a hammer

  - Lack of shielding gas

  - Stress from the arc heating up the metal too fast

  - Stress from cooling the metal too fast

- What is the term for small particles of weld metal adhered to the base metal?

  - Spatter

  - Porosity

  - Crater crack

  - Undercut

- What is the term for excessive throat in a fillet weld bead?

  - Excessive reinforcement

  - Improper reinforcement

  - Excessive concavity

  - Excessive convexity

- What is the term for lack of sufficient weld metal build up in a groove weld?

  - Excessive reinforcement

  - Improper reinforcement

  - Excessive concavity

  - Excessive convexity

- What is it called when the weld metal does not extend deep enough into the root?

  - Excessive concavity

  - Improper reinforcement

  - Incomplete penetration

  - Incomplete fusion

- What causes porosity?

  - Cooling too fast

  - Contaminated base metal

  - Slow travel speed

  - High amperage

- What is NOT a cause of excessive spatter?

  - Improper settings

  - Long arc length

  - Contaminated base metal

  - Cooling too fast

- What can be done to prevent crater crack?

  - Pause at the end of a weld to fill in the crater

  - End the weld quickly to avoid forming a crater

  - End the weld quickly to make a small crater

  - Cool the weld quickly

- Undercut on one toe of the weld bead and overlap on the other toe is a result of what?

  - Improper settings

  - Improper travel angle

  - Improper work angle

  - Improper travel speed

- Which of the following does NOT cause overlap?

  - Fast travel speed

  - Slow travel speed

  - High wire feed speed

  - Improper work angle

- Which of the following is a cause for excessive reinforcement?

  - Fast travel speed

  - Slow travel speed

  - Low wire feed speed

  - Improper work angle

- What is incomplete fusion?

  - When the weld metal forms gas pockets

  - When the weld metal does not extend far enough into the root

  - When the weld metal lacks build up above the base metal

  - When the weld metal does not obtain coalescence with the base metal