# Test - Basic Welding Review & Assessment, Certification for Simulation

Name:

Multiple Choice (chose only the best answer)

- When setting up a welder, what is the work cable is connected to?

  - The positive terminal and the base metal

  - The negative terminal and the base metal

  - The alternating current terminal and the ground

  - The positive terminal and the ground

- Which of the following is NOT part of a fillet weld?

  - Root

  - Throat

  - Reinforcement

  - Toes

- Which of the following is NOT part of a groove weld?

  - Root

  - Throat

  - Face

  - Legs

- What are the two types of angles taken into consideration according to the five basics?

  - Push and travel

  - Work and travel

  - Push and drag

  - Work and push

- What causes porosity?

  - Cooling too fast

  - Contaminated base metal

  - Slow travel speed

  - High amperage

- Undercut on one toe of the weld bead and overlap on the other toe is a result of what?

  - Improper settings

  - Improper travel angle

  - Improper work angle

  - Improper travel speed

- What polarity is GMAW typically performed for the best results?

  - Direct current electrode positive

  - Direct current electrode negative

  - Alternating current

  - Polarity makes no difference in the quality of a weld

- When should the weaving technique NOT be used?

  - To make big welds

  - To cover filler passes

  - To weld in the horizontal position

  - To weld in the vertical position

- What can happen when welding with an improper work angle?

  - Undercut

  - Overlap

  - Excessive spatter

  - All of the above

- How does voltage affect the arc?

  - Determines arc length

  - Determines depth of penetration

  - Shields the arc

  - Voltage has no effect on the arc

- How long should wire stick out be for short circuit GMAW process?

  - 1 - 2 inches

  - 1/2 – 1 inch

  - 1/4 – 1/2 inch

  - 1/8 – 1/4 inch

- Where is the toe of a weld?

  - The imaginary line from the root to the face

  - Where the base metal meets the weld bead

  - Where the base metal meets

  - The imaginary line between the legs

- What should the work angle be for a flat square butt weld?

  - 5-10

  - 0-20

  - 45

  - 90

- What are the parts of a GMAW gun?

  - Liner, diffuser, contact tip, nozzle, trigger

  - Liner, diffuser, guides, nozzle, trigger

  - Liner, guides, drive rolls, contact tip

  - Liner, contact tip, tension adjustment, nozzle, trigger

- Which of the following does NOT cause overlap?

  - Fast travel speed

  - Slow travel speed

  - High wire feed speed

  - Improper work angle

True and False (there is only one correct answer)

- True	False	A groove weld is made with a lap joint.

- True	False	Wire feed speed is associated with amperage.

- True	False	The leg of a fillet weld goes from the root to the toes.

- True	False	Spatter indicates proper settings.

- True	False	A groove weld is made with a butt joint.

- True	False	Fillet and groove are the two main types of welds.

- True	False	A fillet weld is made with an edge joint.

- True	False	If the weld bead is narrow, travel speed is too slow.

- True	False	If there is undercut, the voltage may be too high.

- True	False	Only welding wire comes out of the gun when the trigger is pressed.

- True	False	Square butt joints typically require a gap for good penetration.

- True	False	Corner joints are always fillet welds.

- True	False	Base metal thickness has no effect on proper settings.

- True	False	A poor travel angle can result in lack of penetration.

- True	False	Overlap can indicate too high wire feed speed.